8-bit Arithmetic Logic Unit:

8-bit ALU (Arithmetic Logic Unit) is the core computational block in a CPU that processes 8-bit binary data. This is foundational "mathematical brain" of a computer's Central Processing Unit (CPU). It is a digital electronic circuit responsible for executing all mathematical calculations and logical decisions required by your software and system instructions. An 8-bit Arithmetic Logic Unit (ALU) is the computational core of simple processors, processing 8 bits of data simultaneously. It acts as a digital calculator inside microcontrollers, executing arithmetic (addition, subtraction) and logical (AND, OR, NOT) operations, producing results alongside status flags (e.g., zero, carry, overflow).

ALU is in two primary ways:

1.Arithmetic Operations:

The ALU handles the core math that drives computing.

Basic Math: Addition, subtraction, multiplication, and division.

Counters: Incrementing (adding 1) and decrementing (subtracting 1) values.

2. Logical Operations

The ALU evaluates conditions and manipulates data at the bit level.

Comparisons: Checking if two numbers are equal, greater than, or less than one another. This allows the computer to make decisions and branch program paths.

Bitwise Operations: Using Boolean logic (AND, OR, NOT, XOR) to mask, toggle, or shift specific bits of data.

Status Flags: Outputs specific indicators based on the result, such as:

Zero (Z): Result is all zeros.

Carry (C): An extra bit generated during addition/subtraction, used to link multiple ALUs together.

Overflow (V): The result exceeded the maximum 8-bit capacity.

How It Works:

It is constructed by cascading eight 1-bit ALUs side-by-side. Each 1-bit ALU handles one bit from two separate 8-bit inputs (Operand A and Operand B), linking them together via carry propagation.

1.Inputs: The CPU’s control unit sends instructions and data operands (the numbers to be calculated) from memory into the ALU's temporary registers.

2.Processing: Using hardware logic gates, the ALU manipulates the binary inputs based on the requested instruction.

3.Outputs: The ALU produces the result and updates "status flags" (which indicate conditions like an overflow or a zero result), then sends the final output back to the registers or memory.

Architecture:

An 8-bit ALU is typically constructed by connecting eight 1-bit ALUs in a chain. The hardware routes two 8-bit inputs (A and B), operation controls, and a carry-in value into logic gates and full-adder circuits. Because ALUs are combinational logic circuits, the inputs propagate through the system, producing the final outputs after a specific delay (typically 20–50 nanoseconds depending on the circuit technology)

# Main explanation:

🔥 8-Bit Arithmetic Logic Unit (ALU) using Verilog:

📌 Project Overview:

This project implements an 8-Bit Arithmetic Logic Unit (ALU) using Verilog HDL and verifies its functionality using a custom testbench in Xilinx Vivado.

The ALU is a fundamental component of digital processors, responsible for performing arithmetic and logical operations based on a control signal (sel).

This project was developed as part of my VLSI Design & Verification learning journey.

🚀 Features:

The ALU supports the following operations:

Select (sel)	          Operation

000	                           Addition (A + B)

001	                           Subtraction (A - B)

010	                           Bitwise AND

011	                           Bitwise OR

100	                           Bitwise XOR

101	                           Bitwise NOT (~A)

110	                           Left Shift (A << 1)

111	                           Right Shift (A >> 1)

🧪 Verification:

Test Inputs

A = 20

B = 10

Simulation Results:

Operation	Result:

20 + 10	    30

20 - 10	   10

20 AND 10	   0

20 OR 10	   30

20 XOR 10	   30

NOT 20	   235

20 << 1	   40

20 >> 1	   10

All operations were successfully verified through behavioral simulation.

🎯 Learning Outcomes:

Through this project, I gained hands-on experience in:

- Verilog HDL Coding

- Combinational Logic Design

- RTL Development

- Testbench Writing

- Functional Verification

- Simulation and Debugging

- Vivado Design Flow

🔮 Future Improvements:

- Carry-Out Flag

- Zero Flag Detection

- Overflow Detection

- Parameterized ALU Design

- SystemVerilog Testbench

code:

module alu(

input [7:0] a,

input [7:0] b,

input [2:0] sel,

output reg [7:0] result,

output reg carryout

);

always @(*) begin

carryout = 0;  // Default carryout

case(sel)

3'b000: begin  // Addition

{carryout, result} = a + b;

end

3'b001: begin  // Subtraction

{carryout, result} = a - b;

end

3'b010: begin  // AND

result = a & b;

carryout = 0;

end

3'b011: begin  // OR

result = a | b;

carryout = 0;

end

3'b100: begin  // XOR

result = a ^ b;

carryout = 0;

end

3'b101: begin // NOT (complement)

result = ~a;

carryout = 0;

end

3'b110: begin // Left shift

result = a << 1;

carryout = a[7]; // Carry out is the MSB being shifted out

end

3'b111: begin // Right shift (logical)

result = a >> 1;

carryout = a[0]; // Carry out is the LSB being shifted out

end

default: begin

result = 8'b00000000;

carryout = 0;

end

endcase

end

endmodule

module alu_tb;

reg [7:0] a;

reg [7:0] b;

reg [2:0] sel;

wire [7:0] result;

wire carryout; // Missing wire

alu uut (

.a(a),

.b(b),

.sel(sel),

.result(result),

.carryout(carryout)

);

initial begin

$display("Time\tSel\tA\tB\tResult\tCarry");

$monitor("%0t\t%b\t%d\t%d\t%d\t%b",

$time, sel, a, b, result, carryout);

a = 8'd20;

b = 8'd10;

sel = 3'b000; #10;   // Add

sel = 3'b001; #10;   // Sub

sel = 3'b010; #10;   // AND

sel = 3'b011; #10;   // OR

sel = 3'b100; #10;   // XOR

sel = 3'b101; #10;   // NOT

sel = 3'b110; #10;   // Left Shift

sel = 3'b111; #10;   // Right Shift

$finish;

end

endmodule